A Project report on

CONTEXTUAL BANDIT-BASED ADAPTIVE SYNCHRONIZATION AND RETRAINING OF UAV NETWORK DIGITAL TWINS

project report submitted in partial fulfilment of the requirements for the course

MACHINE LEARNING

25SC2107E

in

A.Y.: 2026-27

by

N. Hasini 	2520040039

Ch. Sai Lahari	              2520040023

Under the Esteemed guidance of

Mr. N Lava Prasad

Assistant Professor,

Department of ECE

ABSTRACT

Unmanned Aerial Vehicle (UAV) networks support monitoring, communication, inspection, and emergency response, but differences in position, speed, and network conditions can cause vehicles to reach a shared target at different times. This project develops a software prototype for multi-UAV arrival-time synchronization and explores Random Forest Regression for speed prediction. The project presentation reports a synthetic dataset of 1,000 scenarios, split into 800 training and 200 testing samples. In that earlier simulation, the reported MAE was 0.0294 and R² was 0.9891. After speed adjustment, the reported arrival times were approximately 44.97, 44.87, and 45.09 seconds, giving an arrival-time spread of about 0.216 seconds. These results are from synthetic simulation and are not real-flight validation. The wider proposed framework uses a Digital Twin as a virtual representation of UAV/network state and plans to apply a contextual bandit to choose actions such as MOVE, WAIT/RETAIN, synchronization, or RETRAIN using context such as traffic, network load, latency, synchronization error, and resource cost. A reward function would balance twin fidelity against update frequency and communication, computation, and energy overhead. The contextual bandit, complete Digital Twin update loop, and automatic adaptive retraining remain future implementation work. The next stage is to evaluate these components against fixed-interval and threshold-based baselines in repeatable dynamic scenarios.

Keywords: UAV network, Digital Twin, Contextual Bandit, Random Forest Regression, Adaptive Synchronization, Retraining.

TABLE OF CONTENTS

| Section | Page |
| --- | --- |
| 1. Introduction | 4 |
| 2. Problem Statement and Motivation | 5 |
| 3. Objectives and Scope | 6 |
| 4. Existing and Proposed Systems | 7 |
| 5. System Model and Architecture | 8 |
| 6. Methodology and Workflow | 9 |
| 7. Dataset and Preprocessing | 10 |
| 8. Algorithm and Implementation | 11 |
| 9. Results and Discussion | 12 |
| 10. Digital Twin, Contextual Bandit and Retraining Plan | 13 |
| 11. Advantages, Applications and Limitations | 14 |
| 12. Future Work and Conclusion | 15 |
| References | 15 |

1. INTRODUCTION

Unmanned Aerial Vehicles are used in inspection, mapping, surveillance, disaster response, and communication support. When several UAVs cooperate on a shared mission, their starting positions, distances, and speeds may differ. If each vehicle follows an independent movement plan, they may reach a target at different times. This arrival-time mismatch can reduce the effectiveness of coordinated observation, formation tasks, and time-sensitive operations.

A Digital Twin (DT) is a virtual representation of a physical asset or system. In a UAV-network setting, the twin may maintain an estimate of each vehicle’s position, speed, battery state, communication quality, and network status. Comparing new telemetry with the virtual state can help identify outdated information and support prediction and planning.

Synchronization involves a trade-off. Frequent updates can increase communication traffic, processing demand, and energy use. Infrequent updates can allow the virtual representation to diverge from the physical system. A context-aware mechanism should update when information is valuable and avoid unnecessary updates when the state is sufficiently stable.

The project begins with a software prototype: estimate arrival times for three UAVs, measure their spread, and explore Random Forest Regression as a speed-prediction component. The wider proposal adds a contextual bandit that can choose actions using current network context and observed outcomes.

1.1 Report organization

The report describes the problem and objectives, compares fixed-policy and adaptive designs, defines the state and synchronization metric, explains the current simulation and model, summarizes reported synthetic results, and identifies work still required to validate the complete framework.

2. PROBLEM STATEMENT AND MOTIVATION

2.1 Problem statement

Multiple UAVs with different initial positions and speeds may reach a common destination at different times. Network traffic, communication delay, resource constraints, and changing conditions can make coordination more difficult. A fixed synchronization interval does not respond to the importance or cost of each update. The project aims to reduce predicted arrival-time mismatch and develop a future policy that balances Digital Twin accuracy against synchronization and retraining costs.

2.2 Challenges

• Different distances and speeds produce unequal arrival times.

• Network load and communication latency can vary over time.

• Frequent twin updates consume bandwidth, computation, and energy.

• Rare updates can leave the twin stale and reduce decision quality.

• A model trained under one scenario may perform poorly after conditions change.

• Synthetic simulation results may not transfer directly to real UAV operations.

2.3 Motivation

The motivation is to combine prediction with context-aware decisions rather than rely only on fixed thresholds. A regression model estimates a useful speed value, while a contextual bandit can learn which action is beneficial under a given context. Prediction estimates a quantity; the decision policy chooses what to do next.

Potential beneficiaries include UAV-network researchers, mission planners, and engineers designing communication-aware coordination systems. The current experiments are software simulations and do not constitute safety-critical flight control.

3. OBJECTIVES AND SCOPE

3.1 Main objective

To develop and evaluate a software framework for adaptive synchronization of multiple UAVs, beginning with arrival-time estimation and Random Forest speed prediction and extending toward context-aware Digital Twin synchronization and retraining.

3.2 Specific objectives

• Represent three UAVs with different starting positions and initial speeds.

• Calculate distance, estimated arrival time, and synchronization spread.

• Train and evaluate a Random Forest Regression baseline on generated scenarios.

• Visualize UAV paths and compare arrival times before and after adjustment.

• Define context variables such as traffic, network load, delay, synchronization error, and cost.

• Design a contextual-bandit action set and reward function for later implementation.

• Plan a measurable retraining trigger and comparison against simple baselines.

3.3 Scope

The current prototype described in the presentation includes a three-UAV simulation, synthetic training data, Random Forest regression, synchronization-error calculations, and visualizations. The contextual bandit, full Digital Twin synchronization loop, and automatic retraining are future implementation items unless completed and tested in a later version.

3.4 Assumptions

• UAV motion is approximated by distance divided by a positive constant speed.

• All simulated UAVs share a common target.

• Reported model metrics refer to the synthetic dataset in the presentation.

• Network context values in demonstrations are illustrative unless sourced from telemetry.

4. EXISTING AND PROPOSED SYSTEMS

4.1 Existing/basic approach

A basic synchronization system estimates arrival times from distance and speed, then adjusts speed or waits according to a predefined rule. A fixed-interval Digital Twin policy updates the virtual state after a set period. These baselines are simple to implement but do not explicitly account for the value and cost of each update.

4.2 Limitations of fixed policies

• A fixed interval may update too often during stable conditions.

• It may update too slowly when the UAV state or network changes rapidly.

• Simple thresholds can oscillate when measurements fluctuate near a threshold.

• A speed predictor alone does not decide whether to move, wait, synchronize, or retrain.

• Without reward-based evaluation, action choices cannot be compared consistently.

4.3 Proposed system

The proposed framework combines a state estimator, speed-prediction model, synchronization monitor, context collector, contextual-bandit decision module, and Digital Twin updater. It estimates the state, predicts or measures synchronization error, observes network conditions, chooses an action, and evaluates the result. Retraining is intended when sustained model-performance degradation is detected.

4.4 Expected difference

| Aspect | Basic approach | Proposed direction |
| --- | --- | --- |
| Decision basis | Fixed interval or thresholds | Current context and learned reward |
| Prediction | Optional speed estimate | Random Forest baseline plus future updates |
| Synchronization | Periodic or rule-based | Adaptive timing based on state and cost |
| Retraining | Manual or fixed schedule | Triggered by measured degradation |
| Evaluation | Arrival-time spread | Spread, twin fidelity, cost, reward and regret |

5. SYSTEM MODEL AND ARCHITECTURE

5.1 UAV geometry

Let UAV i be at pᵢ = (xᵢ, yᵢ), with common target g = (xg, yg). Straight-line distance is dᵢ = √[(xᵢ − xg)² + (yᵢ − yg)²]. If assumed speed is vᵢ > 0, estimated arrival time is tᵢ = dᵢ/vᵢ. This approximation is suitable for a first simulation but omits acceleration, turning, obstacles, wind, and detailed control constraints.

5.2 Synchronization metric

Arrival-time spread is E = maxᵢ(tᵢ) − minᵢ(tᵢ). E = 0 means all estimated arrival times match in the model. A lower spread after adjustment indicates better alignment under the simulation assumptions. It is not by itself a measure of communication quality or Digital Twin fidelity.

5.3 System architecture

| Module | Input | Output / responsibility |
| --- | --- | --- |
| Telemetry/state input | Position, speed, time, optional battery/link data | Current UAV/network state |
| Preprocessing | Raw records and missing values | Clean numeric features |
| Random Forest regressor | Training features and target | Speed estimate |
| Synchronization monitor | Distances and arrival times | Arrival-time spread |
| Context collector | Traffic, load, latency, cost, confidence | Context vector |
| Contextual bandit (planned) | Context and action rewards | MOVE / WAIT / SYNC / RETRAIN decision |
| Digital Twin updater (planned) | New telemetry and chosen update | Updated virtual state |
| Evaluation | Predictions, actions, state errors | Metrics and graphs |

The current code demonstrates telemetry analysis, regression, and synchronization monitoring. The contextual bandit and complete Digital Twin update loop are identified as planned extensions.

6. METHODOLOGY AND WORKFLOW

6.1 Current workflow

• Initialize UAV positions, target, and initial speeds.

• Calculate distances and initial estimated arrival times.

• Calculate initial arrival-time spread.

• Generate synthetic UAV scenarios and prepare features and speed targets.

• Split generated data into training and testing subsets.

• Train a Random Forest Regressor and calculate evaluation metrics.

• Predict speed values for the simulation and calculate adjusted arrival times.

• Plot paths, speed predictions, arrival times, and synchronization spread.

6.2 Proposed adaptive workflow

• Read latest physical or simulated UAV telemetry.

• Update the Digital Twin state estimate.

• Estimate prediction confidence and synchronization error.

• Build context from traffic, network load, latency, cost, and resource state.

• Choose a synchronization action using the contextual bandit.

• Apply the action in simulation and calculate reward.

• Monitor model error and retrain only when a defined trigger is met.

• Compare against fixed-interval and threshold-based baselines.

6.3 Reward design (proposed)

A possible reward is R = −αE − βCupdate − γL − δEnergy − ηCcompute, where E is twin/synchronization error, Cupdate is communication/update cost, L is response latency, Energy is energy consumption, and Ccompute is computation cost. The weights determine the trade-off and must be selected through experiments. This is a proposed design, not a reward function already validated in the current prototype.

7. DATASET AND PREPROCESSING

7.1 Synthetic dataset used in the original prototype

The project presentation reports 1,000 generated UAV scenarios. Features are distance, current speed, arrival time, common arrival time, and synchronization error; the target is required speed. The reported split is 800 training samples and 200 testing samples. Since the dataset is generated from known equations, the feature-target relationship may be highly regular.

| Item | Description |
| --- | --- |
| Dataset type | Synthetic/generated UAV scenarios |
| Number of scenarios | 1,000 (as reported) |
| Input features | Distance, current speed, arrival time, common time, synchronization error |
| Prediction target | Required speed |
| Training samples | 800 |
| Testing samples | 200 |
| Caution | Synthetic evaluation does not establish real-world generalization |

7.2 Public flight telemetry explored separately

A public flight-telemetry CSV was explored as a possible source of measured data. It can support descriptive analysis of time and measured speed and may include altitude, battery, or velocity components depending on the file. It does not supply ground-truth synchronized arrival times for multiple UAVs or verified required-speed labels. It should not be described as the source of the original 1,000-scenario result.

7.3 Preprocessing

• Inspect column names and data types before selecting features.

• Convert time and speed fields to numeric values.

• Remove rows missing required target or feature values.

• Sort telemetry by timestamp before constructing next-sample targets.

• Use a chronological split for time-series forecasting to reduce future-to-past leakage.

• Report row counts after cleaning and exact feature/target definitions.

8. ALGORITHM AND IMPLEMENTATION

8.1 Random Forest Regression

Random Forest Regression combines predictions from multiple decision trees. Each tree is trained on a sampled view of the training data and considers subsets of features at splits. Averaging tree outputs can reduce variance compared with a single tree. It is a useful baseline because it handles nonlinear relationships and requires limited feature scaling.

8.2 Important parameters

| Parameter | Purpose |
| --- | --- |
| n_estimators | Number of decision trees |
| max_depth | Limits tree depth to control complexity |
| min_samples_leaf | Minimum records required at a leaf |
| random_state | Makes the random procedure repeatable |
| Train/test split | Separates fitting from evaluation |

8.3 Evaluation metrics

Mean Absolute Error (MAE) is the average absolute difference between actual and predicted values. Root Mean Squared Error (RMSE) gives larger errors more influence. R² compares model error with a mean-prediction baseline and can be negative when the model performs poorly on the test set. MAE and RMSE units follow the target, such as metres per second when predicting speed.

8.4 Implementation outputs

• Dataset preview, column names, descriptive statistics, and missing-value counts.

• Speed-versus-time, altitude, battery, and numeric correlation plots when fields are available.

• Actual-versus-predicted speed, residual plots, residual histogram, and feature importance.

• UAV path visualization and arrival-time comparison before and after adjustment.

• Summary table of dataset size, model metrics, and simulated synchronization spread.

Each graph should be labelled as measured telemetry, model evaluation, or simulation so that readers do not confuse the sources.

9. RESULTS AND DISCUSSION

The following values are copied from the project-review presentation and refer to the earlier synthetic prototype. They have not been recomputed in this report.

| Metric | Reported value | Meaning / caution |
| --- | --- | --- |
| UAV count | 3 | Three simulated UAVs share one target. |
| Initial speeds | 0.8, 0.5, 1.0 m/s | Different speeds create timing differences. |
| Dataset size | 1,000 | Synthetic scenarios. |
| Train/test split | 800 / 200 | As reported in presentation. |
| MAE | 0.0294 | Reported synthetic test error; units depend on target definition. |
| R² | 0.9891 | Reported synthetic test fit. |
| Adjusted arrival times | 44.97, 44.87, 45.09 s | Reported prototype outputs. |
| Arrival-time spread | 0.216 s | Approximate maximum-minus-minimum spread. |

9.1 Interpretation

The three adjusted arrival times are close to 45 seconds, and their spread is approximately 0.216 seconds. This suggests that the earlier simulated speed-adjustment procedure aligned estimated arrival times for that scenario. It does not demonstrate robust performance across real UAV missions because real dynamics and network effects are not represented by the reported result.

9.2 Graphs for the notebook

• Before/after arrival-time bars for each UAV.

• Synchronization spread before versus after adjustment.

• Measured telemetry speed versus time for public flight-data exploration.

• Actual versus predicted next-sample speed with an identity line.

• Residual error versus time and residual histogram.

• Feature importance, which indicates model reliance rather than causality.

9.3 Fair comparison plan

A fair future comparison should evaluate the same scenarios using fixed-interval updates, threshold-based updates, and the contextual bandit. Report average synchronization/twin error, number of updates, communication cost, latency, energy estimate, cumulative reward, and regret across repeated runs. Do not claim superiority until those tests have been run.

10. DIGITAL TWIN, CONTEXTUAL BANDIT AND RETRAINING PLAN

10.1 Digital Twin state

A practical twin can store timestamped state for each UAV: position, speed, estimated arrival time, battery state if available, and communication variables if measured or simulated. It should also store the last update time and the difference between the twin estimate and the latest observation. State freshness and error can then inform the update decision.

10.2 Contextual bandit

A contextual bandit observes context, selects an action, receives reward, and updates its policy. Possible actions are MOVE, WAIT/RETAIN, SYNCHRONIZE, and RETRAIN. Context may include synchronization error, traffic, network load, latency, model confidence, battery/resource state, and estimated update cost. A genuine bandit requires repeated interaction and reward-based learning; a hand-written score table is only a rule-based demonstration.

10.3 Adaptive retraining

Retraining should be triggered by a defined condition, such as sustained validation error above a threshold, a detected input-distribution change, or reduced prediction confidence. The implementation should preserve validation data and record error before and after retraining. One noisy sample alone is not sufficient evidence of concept drift.

10.4 Remaining implementation

• Implement the bandit and log action, context, reward, and cumulative reward per episode.

• Connect actions to a simulation environment with defined effects.

• Update the Digital Twin after synchronization actions and measure fidelity.

• Test retraining under deliberately changed scenarios.

• Compare policies under the same random seeds and report repeated-run averages.

11. ADVANTAGES, APPLICATIONS AND LIMITATIONS

11.1 Potential advantages

• Provides a numerical synchronization metric based on arrival-time spread.

• Uses Random Forest as a baseline for speed prediction.

• Separates numeric prediction from context-dependent action selection.

• Creates a pathway to reduce unnecessary Digital Twin updates.

• Supports repeatable software experiments and visualization before hardware testing.

11.2 Potential applications

• Coordinated UAV inspection and infrastructure monitoring.

• Multi-UAV surveillance and area coverage.

• Disaster-response mapping and time-sensitive observation.

• Research on communication-aware UAV coordination and Digital Twin maintenance.

11.3 Limitations

• Reported regression metrics come from synthetic data and may be optimistic.

• The simple model assumes straight-line travel at constant speed.

• Wind, obstacles, acceleration, battery constraints, and flight safety limits are not modeled in the basic calculation.

• Public telemetry does not provide ground-truth multi-UAV synchronization labels.

• The contextual bandit, full Digital Twin update loop, and automatic retraining remain proposed extensions until implemented and evaluated.

• Illustrative context values or action scores must not be presented as real-dataset measurements.

12. FUTURE WORK AND CONCLUSION

12.1 Future work

• Develop a simulator varying traffic, network load, latency, packet loss, and resource availability.

• Implement a contextual bandit with a specified action set, exploration strategy, and reward.

• Integrate speed predictions with the UAV synchronization simulator.

• Build a Digital Twin state store with timestamps, freshness, and state-error metrics.

• Implement adaptive retraining using a validation-window or drift-based trigger.

• Compare with fixed-interval and threshold-based baselines over repeated scenarios.

• Report fidelity, synchronization, update frequency, cost, latency, reward, and regret.

12.2 Conclusion

This project establishes a software prototype for multi-UAV arrival-time synchronization and explores Random Forest Regression as a speed-prediction component. The project presentation reports a final arrival-time spread of approximately 0.216 seconds for one three-UAV synthetic scenario, with MAE 0.0294 and R² 0.9891 reported for the synthetic test set. These results are an initial baseline and do not establish real-world performance. The next step is to implement and evaluate the contextual bandit, Digital Twin synchronization policy, and adaptive retraining loop under dynamic conditions, separating measured data from simulation.

REFERENCES

Group 7, “Contextual Bandit-Based Adaptive Synchronization and Retraining of UAV Network Digital Twins,” project review presentation, Machine Learning (25SC2107E), A.Y. 2026–27.

Group 7, “Contextual Bandit-Based Adaptive Synchronization and Retraining of UAV Network Digital Twins,” project abstract document.

Project Report Template supplied for the Machine Learning course, Department of Electronics & Communication Engineering.

Project PPT Components, supplied guidance document.